Assignment Module 8: CCNA-Routing and Switching

- Explain Switch.

- What is a switch?

A switch is a network device that connects multiple devices (PCs, printers, servers) within a LAN and forwards data only to the intended device using MAC addresses.

- How a switch Works

- works at layer 2 (data link layer) of the OSI model

- Learns MAC addresses of connected devices

- Stores them in a MAC address table

- Sends data only to the destination port, not everyone

- Features

- Reduces network traffic

- Faster than hubs

- Support VLANs, security, and full-duplex communication

- Real-world example

Think of a courier office

- Every desk has a unique address

- Parcel goes only to the correct desk, not to all desks

That’s exactly how a switch sends data.

- Explain the switch boot sequence

- What is the boot sequence?

The boot sequence is the step-by-step process a switch follows when it is powered on.

- Switch boot step

- Power ON

- Switch receives power

- POST (Power-on self-test)

- Checks CPU, RAM, FLASH memory

- Boot loader

- A small program stored in ROM

- Initializes hardware

- Load Cisco IOS

- ISO loaded from FLASH memory into RAM

- Load startup configuration

- Reads configuration from NVRAM

- Id not found – enter setup mode

- Real-world example

Like starting your mobile phone

- Hardware check ---> OSI loads ---> your setting appear

- Three methods to access the switch

- Console access

- uses console cables

- used for initial configuration

- no network required

- Example: Like using a USB cable to set up a new phone

- Telnet access

- Remote access using IP addresses

- Uses TCP port no.23

- Net secure (password in plain text)

- Example: Calling someone without encryption

- SSH access

- Secure remote access

- Encrypted communication

- Uses TCP port no. 22

- Example: WhatsApp call (secure & encrypted)

- Explain and configure Cisco IOS

- What is Cisco IOS?

Cisco IOS (internetwork operating system) is the software that runs Cisco switches and routers.

- Functions of IOS

- Routing and switching

- Security

- Device management

- Network monitoring

- Basic IOS configuration (switch)

Switch> enable

Switch# configure terminal

Switch(config)# enable password cisco

Switch(config)# line console 0

Switch(config-line) # password admin

Switch(config-line) # login

Switch(config)# exit

Switch(config)# exit

SW1 write memory

- Real-world example

iOS is like Windows or Android OS

Without it, the device is useless

- Explain switch port

- What is a switch port?

A switch port is a physical interface on a switch where devices are connected using Ethernet cables.

- Types of switch ports

- Access port- connect end devices (pc, printer)

- Trunk port – carries multiple VLANs

- Uplink port – connect switches

- Port functions

- Sends and receives data

- Can be enabled/disabled

- Can be assigned to VLANs

- Real-world example

Switch ports are like power sockets

- Each socket connects one device

- Data flows only through that socket

- Enable secret (password) is hashed using the _____ algorithm

- MD5

- AH

- PSK

- ESP

- WPA2

Answer: a) MD5

- An engineer connects to Router R1 and issues a show ip ospf neighbor command. The status of neighbor 2.2.2.2 lists FULL/BDR. What does the BDR mean?

- R1 is an Area Border Router.

- R1 is a backup designated router.

- Router 2.2.2.2 is an Area Border Router.

- Router 2.2.2.2 is a backup designated router.

Answer: d) Router 2.2.2.2 is a backup designated router.

- Which command is used to view the neighbor discovery table on a PC?

- show IPv6 neighbor

- show IPv6 neighbors

- show IPv6 neighbors

- netsh interface ipv6 show neighbors

Answer: d) netsh interface ipv6 show neighbors

- What type of variable is being shown? Routers = [R1,R2,R3]

- List

- Dictionary

- Simple

- Unsigned integers

Answer: a) List

- Identify the fields in an IPv4 header. (Choose three)

- Host components

- Time to live

- Source address

- Destination address

- Network address

Answer: b) time to live, c) secure address, d) destination address

X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X

THANK YOU