Assignment Module 6: Network Security, Maintenance, and Troubleshooting Procedures

Section 1: Multiple Choice

- What is the primary purpose of a firewall in a network security infrastructure?

- Encrypting network traffic

- Filtering and controlling network traffic

- Assigning IP addresses to devices

- Authenticating users for network access

Answer: b)	Filtering and controlling network traffic

- What type of attack involves flooding a network with excessive traffic to disrupt normal operation?

- Denial of Service (DoS)

- Phishing

- Spoofing

- Man-in-the-Middle (MitM)

Answer: a)	Denial of Service (DoS)

- Which encryption protocol is commonly used to secure wireless network communications?

- WEP (Wired Equivalent Privacy)

- WPA (Wi-Fi Protected Access)

- SSL/TLS (Secure Sockets Layer/Transport Layer Security)

- AES (Advanced Encryption Standard)

Answer: b)	WPA (Wi-Fi Protected Access)

- What is the purpose of a VPN (Virtual Private Network) in a network security context?

Answer:

A VPN (Virtual Private Network) keeps your online activity private by encrypting your internet traffic and routing it through a secure tunnel. This hides your real IP address and protects your data from hackers – especially when you’re using public Wi-Fi. In simple terms, a VPN gives you privacy, security, and safe remote access to the internet from anywhere.

Section 2: True or False

- True or False: Patch management is the process of regularly updating software and firmware to address security vulnerabilities and improve system performance.

Answer: True

- True or False: A network administrator should perform regular backups of critical data to prevent data loss in the event of hardware failures, disasters, or security breaches.

Answer: True

- True or False: Traceroute is a network diagnostic tool used to identify the route and measure the latency of data packets between a source and destination device.

Answer: True

Section 3: Short Answer

- Describe the steps involved in conducting a network vulnerability Assignment.

Answer:

- Define the scope:

Decide which system, networks, applications, and devices will be tested. A clear scope ensures no critical components are left out.

- Identify & analyze risks:

List valuable assets (servers, databases, IoT devices) and assess which are more sensitive; high-risk assets get priority.

- Develop policies & producer:

Create rules of scanning timings, tools to be used, permissions, and how reports will be handled. This avoids disruption and keeps the process controlled.

- Choose the type of scan:

Select internal scans (inside the network) or external scans (from outside, like a hacker). This ensures both internal and external threats are covered.

- Configure & run the scan:

Use automated tools such as Nessus, Qualys, or OpenVAS to detect vulnerabilities like outdated patches, weak passwords, or misconfiguration.

- Evaluate & prioritize risk:

Not all vulnerabilities are equally dangerous. Classify them into critical, high, medium, and low based on impact and urgency.

- Interpret & report results:

Convert technical findings into simple business impact and recommend fixes such as patching, configuration changes, MFA, or firewall rules.

Section 4: Practical Application

- Demonstration of how to troubleshoot network connectivity issues using the ping commands

Answer:

- Introduction:

Ping is a basic network diagnostic tool that uses ICMP Echo request and echo reply messages to check connectivity between two devices. It helps identify where a communication failure occurs in the network.

- Ping the loopback address (127.0.0.1):

- Test the local computer’s TCP/IP stack.

- If the fails, the problem is inside the system and not in the network.

- Ping your own IP address:

- example: ping 192.168.x.x

- verifies that the network interface card (NIC) is working and properly configured.

- Failures indicate NIC or driver issues.

- Ping the default Gateway (Router):

- Example: ping 192.168.1.1

- Confirms whether the device can communicate with the router.

- Failures suggest cable issues, Wi-Fi disconnection, or router problems.

- Ping an external IP (e.g., 8.8.8.8):

- Test internet connectivity beyond the local network.

- If the gateway works but fails, the issue may be with the ISP or routing.

- Ping a domain name (e.g., www.google.com):

- Checks both connectivity and DNS resolution.

- If IP ping works but the domain fails – DNS server is misconfigured or not responding.

- Conclusion:

Using ping step-by-step helps isolate faults at each level- local PC, NIC, router, ISP, or DNS. This makes ping one of the most important tools for quick and effective network troubleshooting.

Section 5: Essay

- Discuss the importance of regular network maintenance and the key tasks involved in maintaining network infrastructure.

Answer:

Regular network maintenance is the routine process of checking, updating, and improving network devices and software to ensure the network remains reliable and efficient. It helps avoid sudden failures, strengthens security, and keeps the system performing smoothly.

- Prevents downtime

Regular inspection of routers, switches, and cables helps detect faults early, reducing unexpected outages that affect users and business operations.

- Enhancer security

Maintenance includes installing security patches, updating firmware, reviewing firewall rules, and removing unauthorized devices. These steps protect the network from malware and hacking attempts.

- Improves performance

Monitoring bandwidth, latency, and device load helps identify bottlenecks. This ensures fast data transfer, smooth video calls, and efficient use of network resources.

- Extends device lifespan

Cleaning hardware, replacing damaged cables, and checking ventilation ensure that networking equipment lasts longer and works reliably.

- Protect important data

Regular backups of configurations and critical files help recover the network quickly after failures, attacks, or human mistakes.

- Ensures proper access controls

Maintenance involves reviewing user accounts, passwords, and access rights to prevent unauthorized access and misuse of network resources.

- Supports compliance and documentation

Industries often require a network to follow security standards. Documenting charges, audits, and maintenance logs ensures compliance and provides references for future troubleshooting.

- Which of the following best describes the purpose of a VPN (Virtual Private Network)?

- Encrypting network traffic to prevent eavesdropping

- Connecting multiple LANs (Local Area Networks) over a wide area network (WAN)

- Authenticating users and controlling access to network resources

- Reducing latency and improving network performance

Answer: a) Encrypting network traffic to prevent eavesdropping

X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X-X

THANK YOU